FAST EcoBuild

Maintenance Procedures

CLEANING

Wash down with low pressure water & soft brush. Do not use aggressive detergents.

FIXING

When replacing rainwater pipes or any other attachment to the façade, of FAST  External Insulated Render, drill 150mm deep to ensure that the fixing is anchored to the masonry structure of the wall. Insert a 140mm screw to provide secure fixing.

WINDOW REPLACEMENT

Your windows are securely sealed to the FAST  External Insulation Render.

By using a sharp blade, at the interphase of the render & the window, release the seal. Then remove the windows from inside the house.

Remove all traces of existing seal & use FAST  Neoprene Tape Seal to attach to new window frame & seal with FAST Silicone Seal.

TECHNICAL ADVICE

Contact:  086 2572944